#### SAP Printing

#### Device type Configuration

for

Dragon

(ZHP4DRAG)

Updated 4th January 2007

- Overview

This chapter introduces our approach to supporting printing to our devices under SAP R/3.

Sharp printer controllers support the PCL, PJL and/or PostScript printer language.

The customer can print to Sharp devices with any suitable SAP R/3 standard device type, such as HPLJ4. However, using the HPLJ4 device type it is hard to use the range of available features, like additional input and output trays, finishing options, stapling and punching. Some settings could be configured on the device's front panel, however this is only for permanent changes, but is not recommended on a job by job basis. For these reasons, we have developed device types that incorporate all the option available on Sharp’s devices.

The device types differ in the class of devices they support, the printer language they are based on, and the features supported.

Each device type is "generic" when shipped.

For each device setting the "generic" code contains initially a default command sequence, and other available options commented out.

The shipped device type has to be used as a template for creating the actual device type required.

The accompanying information provides the necessary details the customer’s SAP administrator will require to configure the generic device type.

See below for the contents.

2. Device type settings

To modify the PJL command, select the device type then change>formats.  Select the format you need to modify and double click the ‘printer initialisation’ button. Scroll down to the relevant command.

The options shown in this document is taken from the DINA4 format but these settings are available in all formats. This device type is based on the SAP HPLJ4

The commands selected will depend on the options fitted to the machine

Tray Selection

Automatic is selected by default. To select another paper tray remove the # on desired tray and make sure all other option are deselected with the # symbol

Default settings				         Tray 3 Selected

| # Set Input Tray @PJL SET MEDIASOURCE = AUTOMATIC\r\n # @PJL SET MEDIASOURCE = TRAY1\r\n # @PJL SET MEDIASOURCE = TRAY2\r\n # @PJL SET MEDIASOURCE = TRAY3\r\n # @PJL SET MEDIASOURCE = TRAY4\r\n # @PJL SET MEDIASOURCE = LCC\r\n # @PJL SET MEDIASOURCE = MANUALFEED\r\n | # Set Input Tray # @PJL SET MEDIASOURCE = AUTOMATIC\r\n # @PJL SET MEDIASOURCE = TRAY1\r\n # @PJL SET MEDIASOURCE = TRAY2\r\n @PJL SET MEDIASOURCE = TRAY3\r\n # @PJL SET MEDIASOURCE = TRAY4\r\n # @PJL SET MEDIASOURCE = LCC\r\n # @PJL SET MEDIASOURCE = MANUALFEED\r\n |
| --- | --- |

Booklet Printing

For booklet printing all options need to be selected by removing the # symbol. This makes it easier to configure the device When options such as booklet are required, all the option listed in the booklet section need type to print booklets, the paper source tray has not included here, this needs to be selected in set ‘MEDIASOURCE’ area

Default Settings				        Booklet Enabled

| # Set Booklet Mode # @PJL SET JOBBOOKLET = ON\r\n # @PJL SET OUTTRAY = TRAY4\r\n # @PJL SET DUPLEX = ON\r\n # @PJL SET BINDING = LONGEDGE\r\n # @PJL SET JOBSTAPLE = SADDLE\r\n # @PJL SET PAGEIMAGEINFO = TWOUPPAMPHLET\r\n # @PJL SET ORIENTATION = PORTRAIT\r\n | # Set Booklet Mode @PJL SET JOBBOOKLET = ON\r\n @PJL SET OUTTRAY = TRAY4\r\n @PJL SET DUPLEX = ON\r\n @PJL SET BINDING = LONGEDGE\r\n @PJL SET JOBSTAPLE = SADDLE\r\n @PJL SET PAGEIMAGEINFO = TWOUPPAMPHLET\r\n @PJL SET ORIENTATION = PORTRAIT\r\n |
| --- | --- |

To print booklets select a paper source tray with paper loaded for short edge feed

# Set Input Tray

@PJL SET MEDIASOURCE = TRAY3\r\n

Enable Staple Mode

Default					         One staple enabled to finisher tray2

| # Set Staple Mode(enable one of the staple-functions and TRAY2 or TRAY3) # @PJL SET JOBSTAPLE = STAPLELEFT\r\n # @PJL SET JOBSTAPLE = STAPLEBOTH\r\n # @PJL SET OUTTRAY = TRAY2\r\n # @PJL SET OUTTRAY = TRAY3\r\n | # Set Staple Mode(enable one of the staple-functions and TRAY2 or TRAY3) @PJL SET JOBSTAPLE = STAPLELEFT\r\n # @PJL SET JOBSTAPLE = STAPLEBOTH\r\n @PJL SET OUTTRAY = TRAY2\r\n # @PJL SET OUTTRAY = TRAY3\r\n |
| --- | --- |

3. OPTIONS

Most commands in the device type are standard to the basic AR-M550/620/700. This section describes the other PJL commands available and the options they relate to.

| Function | PJL Option Name | PJL Option Value | Means | Option |
| --- | --- | --- | --- | --- |
| Paper Source | MEDIASOURCE | LCC | Select LCC | AR-LC6 |
| Job Finishing | OUTTRAY | TRAY2 | Job is output to the second tray with job separation | AR-F15 or AR-F16 |
| Job Finishing | OUTTRAY | TRAY3 | Job is output to offset of finisher | AR-F15 or AR-F16 |
| Job Finishing | OUTTRAY | TRAY4 | Job is output to offset of saddle finisher | AR-F16 |
| Staple Control | JOBSTAPLE | STAPLELEFT | Staple in corner | AR-F15 or AR-F16 |
| Staple Control | JOBSTAPLE | STAPLEBOTH | Two staples | AR-F15 or AR-F16 |
| Staple Control | JOBSTAPLE | SADDLE | Saddle Stitch | AR-F16 |